# SAFETY DATA SHEET

# Cerium Oxide

According to WHS Regulations and GHS Revision 7 (Australia)

## Section 1: Identification

Product Name: Cerium Oxide
Synonyms: Ceric Oxide, Cerium Dioxide
Chemical Name: Cerium(IV) oxide
CAS Number: 1306-38-3
Recommended Use: Polishing compound, catalyst, glass additive
Restrictions on Use: Not for pharmaceutical or food use

Supplier Details (Australia): Roy Lehmann
Company Name: Roys Rocks
Address: Online only at www.roysrocks.com
                  PO Box 13080, University of Adelaide, South Australia, 5005, Australia
ABN: 81 841 683 279
Email: roys.rocks@yahoo.com
Emergency Phone Number: Poisons Information Centre (131126)

## Section 2: Hazard Identification

Classification of the substance (GHS - Australia):
Not classified as hazardous according to the criteria of Safe Work Australia.

Label Elements:
Signal Word: None
Hazard Pictogram(s): None
Hazard Statement(s): Not classified as hazardous
Precautionary Statement(s):
P261: Avoid breathing dust
P264: Wash hands thoroughly after handling
P280: Wear protective gloves/eye protection
P304 + P340: IF INHALED: Remove person to fresh air and keep comfortable for breathing
P305 + P351 + P338: IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses if present and easy to do

Other Hazards:
Dust may cause mechanical eye or respiratory irritation
May aggravate respiratory conditions if inhaled in large amounts

## Section 3: Composition / Information on Ingredients

| Ingredient | CAS Number | Proportion |
| --- | --- | --- |
| Cerium Oxide (CeO₂) | 1306-38-3 | >99% |

## Section 4: First Aid Measures

Inhalation: Remove to fresh air. Seek medical attention if symptoms persist.
Skin Contact: Wash thoroughly with soap and water.
Eye Contact: Rinse cautiously with water for at least 15 minutes. Remove contact lenses if present.
Ingestion: Rinse mouth. Do not induce vomiting. Seek medical attention if unwell.
Advice to Doctor: Treat symptomatically.

## Section 5: Firefighting Measures

Suitable Extinguishing Media: Water spray, dry chemical, foam, carbon dioxide
Hazards from Combustion: Inert material; may release cerium oxides in high-temperature fires
Precautions for Firefighters: Wear self-contained breathing apparatus and full protective gear

## Section 6: Accidental Release Measures

Personal Precautions: Avoid dust formation. Use PPE including respirator if ventilation is inadequate.
Environmental Precautions: Prevent material entering sewers, watercourses, or soil.
Methods for Clean-Up: Sweep or vacuum without creating dust. Collect in labeled, closed containers for proper disposal.

## Section 7: Handling and Storage

Handling: Avoid inhalation of dust. Handle in well-ventilated area. Use PPE.
Storage: Store in a dry, cool, well-ventilated location. Keep container tightly closed. Protect from moisture.

## Section 8: Exposure Controls / Personal Protection

Exposure Standards (Safe Work Australia):
Substance: Cerium compounds (as Ce)
TWA (8-hour): 1 mg/m³
STEL: None established

Engineering Controls: Use local exhaust ventilation to control dust levels

Personal Protective Equipment (PPE):
Eye Protection: Safety goggles or face shield
Skin Protection: Gloves, lab coat or protective clothing
Respiratory Protection: P2 particulate respirator if dust is generated

## Section 9: Physical and Chemical Properties

Appearance: Pale yellow to white powder
Odour: Odourless
Melting Point: ~2400 °C
Boiling Point: Sublimes
Solubility in Water: Insoluble
pH: Not applicable
Vapour Pressure: Not applicable
Flash Point: Not applicable
Auto-Ignition Temperature: Not flammable
Relative Density: ~7.2 g/cm³

## Section 10: Stability and Reactivity

Stability: Stable under recommended storage conditions
Reactivity: Inert under normal conditions
Incompatible Materials: Strong acids and strong oxidisers
Hazardous Decomposition Products: Metal oxides at high temperatures

## Section 11: Toxicological Information

Likely Routes of Exposure: Inhalation, eye/skin contact
Acute Effects: Dust may irritate respiratory tract and eyes
Chronic Effects: Prolonged inhalation of dust may affect lungs
Carcinogenicity: Not listed as carcinogenic by IARC, NTP, or Safe Work Australia
LD50/LC50: No data available for cerium oxide specifically; low toxicity expected

## Section 12: Ecological Information

Ecotoxicity: Not classified as hazardous to the environment
Persistence and Degradability: Inorganic and not biodegradable
Bioaccumulation Potential: Not expected to bioaccumulate
Mobility in Soil: Low mobility due to insolubility

## Section 13: Disposal Considerations

Disposal Methods: Dispose in accordance with local environmental regulations.
Waste Code: Not classified as hazardous waste under Australian legislation.
Packaging Disposal: Rinse and dispose of empty containers safely.

## Section 14: Transport Information

UN Number: Not regulated
Proper Shipping Name: Not regulated for transport
Transport Hazard Class: None
Packing Group: None
Special Precautions: Avoid creating dust during transport

## Section 15: Regulatory Information

Australia (AIIC): Listed on the Australian Inventory of Industrial Chemicals (AIIC)
Poisons Schedule (SUSMP): Not scheduled
Workplace Exposure Standards: Refer to Section 8

## Section 16: Other Information

Date of Preparation: 22/05/2025
SDS Version: 2.0
Prepared By: Roy Lehmann/Roys Rocks

Disclaimer:
The information contained herein is believed to be accurate and up to date. However, it is the responsibility of the user to ensure suitability for their own use. No warranty is expressed or implied.